Transportation Research Part C 77 (2017) 306–328

Contents lists available at ScienceDirect

Transportation Research Part C

journal homepage: www.elsevier.com/locate/trc

Forecasting short-term subway passenger flow under special events scenarios usingmultiscale radial basis function networksq

Yang Li a, Xudong Wang a, Shuo Sun b, Xiaolei Ma c,b,⇑, Guangquan Lu c,b a School of Automation Science and Electrical Engineering, Beihang University, Beijing 100191, China

b School of Transportation Science and Engineering, Beijing Key Laboratory for Cooperative Vehicle Infrastructure System and Safety Control, Beihang University, Beijing 100191, China

c Beijing Advanced Innovation Center for Big Data and Brain Computing, Beihang University, China

article info

Article history:

Received 17 February 2016

Received in revised form 18 January 2017 Accepted 6 February 2017

Available online 16 February 2017

Keywords:

Matching pursuit orthogonal least squares (MPOLS)

Nonlinear system identification

Radial basis function (RBF) networks Smart card data

Subway passenger flow prediction Special events

1. Introduction

abstract

Reliable and accurate short-term subway passenger flow prediction is important for pas sengers, transit operators, and public agencies. Traditional studies focus on regular demand forecasting and have inherent disadvantages in predicting passenger flows under special events scenarios. These special events may have a disruptive impact on public transporta tion systems, and should thus be given more attention for proactive management and timely information dissemination. This study proposes a novel multiscale radial basis func tion (MSRBF) network for forecasting the irregular fluctuation of subway passenger flows. This model is simplified using a matching pursuit orthogonal least squares algorithm through the selection of significant model terms to produce a parsimonious MSRBF model. Combined with transit smart card data, this approach not only exhibits superior predictive performance over prevailing computational intelligence methods for non-regular demand forecasting at least 30 min prior, but also leverages network knowledge to enhance predic tion capability and pinpoint vulnerable subway stations for crowd control measures. Three empirical studies with special events in Beijing demonstrate that the proposed algorithm can effectively predict the emergence of passenger flow bursts.

2017 Elsevier Ltd. All rights reserved.

Subway transportation systems are adopted worldwide as effective countermeasures to mitigate the adverse effects of rapid urbanization and traffic congestion (Ma et al., 2015). In the past decades, the construction of urban rail transits in both developed and developing countries has been increasing, and the expanding subway networks have stimulated a sharp rise in ridership (Zhang and Wang, 2014). Taking the Beijing subway system as an example, the average daily ridership in 2012 was 6.74 million passengers compared with the 1.19 million passengers in 2000 (Si et al., 2015). The ridership explosion has triggered a series of issues, such as crowdedness in trains and the insufficient capacity of subway facilities (Zhong et al., 2016). These phenomena became very common in metropolitan cities, including Beijing, Shanghai, and Tokyo (Ma et al., 2012). In particular, passengers in Beijing may not be able to board trains successfully due to overcrowdedness during peak

q This article belongs to the Virtual Special Issue on ‘‘Smart cards, big data and travel behaviour”.

⇑ Corresponding author at: School of Transportation Science and Engineering, Beijing Key Laboratory for Cooperative Vehicle Infrastructure System and Safety Control, Beihang University, Beijing 100191, China.

E-mail addresses: liyang@buaa.edu.cn (Y. Li), wxd_1024@foxmail.com (X. Wang), sunshuo@buaa.edu.cn (S. Sun), xiaolei@buaa.edu.cn (X. Ma), lugq@buaa.edu.cn (G. Lu).

http://dx.doi.org/10.1016/j.trc.2017.02.005

0968-090X/  2017 Elsevier Ltd. All rights reserved.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 307

hours. To alleviate the burden of high passenger demands, passengers are routinely restricted at certain subway stations and have to line up for boarding, or are even prohibited from entering the stations during peak hours. One possible solution to this issue is to provide proactive passenger flow forecasting information for passengers to adjust their travel paths, modes, and departure times. Reliable passenger flow estimation can also benefit transit operators in optimizing service schedules.

Existing literature on subway passenger flow prediction can generally be categorized into long-term and short-term pre dictions. Long-term passenger flow prediction essentially estimates future travel demands through the four-step transporta tion planning model or regression techniques (Horowitz, 1984). Regression models are required to establish a relationship between subway ridership and a series of influential factors, such as demographic, economic, and land-use information (Taylor et al., 2009; Chan and Miranda-Moreno, 2013). For short-term passenger flow prediction, statistical and computa tional intelligence-based models are extensively studied (Karlaftis and Vlahogianni, 2011; Ma et al., 2015; Vlahogianni et al., 2014). Generally, these models can be classified as time series (Ma et al., 2013; Xue et al., 2015), neural network (Tsai et al., 2009), support vector machine (Sun et al., 2015; Wang et al., 2015), and empirical mode decomposition (Jiang et al., 2014). Although these methods can capture the subtle and regular fluctuation of subway passenger flow to some extent, they are not designed to forecast non-regular demands due to special events (Pereira et al., 2015). These special events include concerts, sporting events, parades, and other public events that may attract a large group of people. When these events occur, a significant rise in passenger demand can be observed; such passenger flow irregularities are very dif ficult to forecast and manage because of poor information availability (Pereira et al., 2015). Determining the spatial and tem poral distributions of passenger flows in advance is crucial for emergency response and stampede prevention (Zhong et al., 2015) and is considered a more effective countermeasure than reactive control and evacuation strategies. With reliable pas senger flow information, transit operators can adopt several proactive measures to avoid overcrowding, such as bus feeder allocation, warning information dissemination, and temporary closure of stations. However, predicting irregular travel demands under special events is much more difficult than regular demand forecasting, because special events are nonrecur ring and leave scant prior information for pattern recognition and machine learning models. Therefore, very limited effort has been made in this domain, with three exceptions. Kuppam et al. (2013) utilized the traditional four-step model to demonstrate the demand for special events. Pereira et al. (2015) developed a neural network algorithm to predict public transit arrivals for special events using social media and smart card data, as well as demonstrated the quality of transport predictions under special events. Ni et al. (2017) developed a hashtag-based event detection algorithm to detect high subway passenger flow based on social media data, and then proposed a parametric and convex optimization-based approach to forecast the passenger flow.

In the context of subway systems, predicting passenger flows under special events deserves more attention, especially in metropolitan cities where a large portion of the inhabitants take the subway for commuting (Zhou et al., 2014). Alternatively, a number of statistical and non-statistical model approaches have been developed, such as autoregressive moving average (ARMA), artificial neural networks (ANNs), support vector regression, linear regression, and radial basis functions (Pereira et al., 2015), to solve the short-term prediction of time series under special events. Among these methods, ANN has partic ularly gained greater attention due to its outstanding capabilities of capturing the potential nonlinear dynamical properties between input–output data. However, conventional ANNs utilized have issues with overtraining, local optima, and high computational burden (Zhang et al., 2014). The ARMA approach has also attracted substantial attention in forecasting time series to overcome these problems. Recently, some variants of ARMA models were developed to solve the issue of time series prediction, including simplified autoregressive, autoregressive integrated moving average (Khashei et al., 2012), and ARMA (Rojas et al., 2008). However, these models commonly require high model orders to accommodate stochastic variations of forecasting time series. In addition, these data sequence models are essentially linear. Thus, they may not adapt well in revealing the characteristics of the stochastic nature and uncertain nonlinear dynamics of time series data from special events, because most of the real applications are complex and nonlinear in nature (De Gooijer and Hyndman, 2006).

To tackle these problems, we proposed an identification approach for detecting and forecasting subway passenger flows under special events. This approach does not require any prior information regarding the events and only relies on limited observational subway passenger flow data. To achieve this objective, we proposed a novel class of multiscale radial basis func tion (MSRBF) networks to represent the underlying dynamics of the abnormal subway network. Generally, a conventional single scale or kernel width RBF network means that all basis functions have only a common single scale, or each basis func tion has a single individual scale. By contrast, the new RBF network in this work is employed as a number of multiscale basis functions, where each basis function contains multiple scale kernel widths. The detailed procedure of constructing an MSRBF network is as follows. First, in the MSRBF network, some unsupervised clustering algorithms, such as the fast fuzzy c-means clustering approach, are initially adopted to pre-cluster and determine the positions or centers of the basis functions. Second, for each selected center, the associated kernel widths (scales) are determined heuristically, and the selected centers and scales are then restricted to a fixed grid. Finally, an MSRBF network is converted into a model with linear parameters. An effective matching pursuit orthogonal least squares (MPOLS) algorithm (Billings and Wei, 2005), together with a generalized cross-validation (GCV) criterion (Li et al., 2011b, 2011a), is then applied to train the MSRBF network. A parsimonious model that only includes a relatively small number of regressor terms is likewise achieved and further used to predict the abnormal subway passenger flow at least 30 min ahead. Different from the majority of existing literature on passenger flow prediction, one of the advantages of the proposed algorithm is that it is capable of predicting station-level passenger flows by consid ering network knowledge and of ranking the relative contributions of other subway stations on prediction. Before the sudden rise of passenger flows at a certain station can be observed, there may be several other stations with high fluctuations of

308 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

passenger flows. The irregularities of passenger flows for these stations are highly related to the occurrence of passenger flow burst at the targeted station. Thus, they can be used as inputs for prediction. In this case, both spatial and temporal features of passenger flows are considered in our model to enhance the predictive capability under special events scenarios.

The remainder of this paper is organized as follows. Section 2 describes the datasets used in this study. Section 3 presents the proposed approach and the acquired methodology. Section 4 provides the methodology applied for the experimental comparison with other prevailing prediction algorithms. Finally, Section 5 discusses the conclusion and future works.

2. Data source

The Beijing subway system comprises 18 lines and 319 stations with 527 km of subway network (Si et al., 2015). Fig. 1 presents the layout of the Beijing subway network. Smart card data are used to generate subway passenger flows. In 2006, the entire subway system in Beijing began to accept the smart card. The fare was initially fixed, but it then switched to distance-based in 2014. In 2014, cardholders who spent more than 100 RMB on subway fare receive credit rebates (up to 27% discount) for the following month. This policy attracts a great number of passengers taking the subway, with daily pas senger flows of 9.2786 million. Passengers are required to tap their cards at the gate before entering and exiting the subway station (Long and Thill, 2015), and both boarding and alighting information (e.g., times and stops) are recorded (Ma et al., 2013). For each station, the boarding and alighting passenger flows can be estimated and aggregated every 15 min.

Passenger flow data from July 1, 2012 to October 31, 2012 were used as the dataset in this study. A total of 19 stations were selected as the candidate stations for prediction and are displayed in Fig. 1. The abbreviation of each station is listed in Table 1. These stations are either important transfer hubs or stations with very high passenger demands. Their average daily boarding and alighting passenger flows are calculated in Table 1. Among these stations, WKS, DZM, and HDHZ stations are the three target stations for prediction. The WKS station is adjacent to the MasterCard Center, which is an indoor stadium with a capacity of 18,000 and frequently hosts important sports games and vocal concerts. There was one pop-star concert on July 21, 2012 (Saturday, 7:00 PM). Before the performances began, the crowd flowed out of the WKS station and increased

Fig. 1. Beijing subway network (The WKS station is marked in red, the DZM station is marked in blue, the HDHZ station is marked in green, and the remaining stations are marked in black.) (For interpretation of the references to colour in this figure legend, the reader is referred to the web version of this article.)

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 309

Table 1

Average daily boarding and alighting passenger demands for 19

subway stations.

Station name Boarding Alighting

Beijing Station (BS) 34,261 36,906

Chaoyangmen (CYM) 34,374 35,529

Dawanglu (DWL) 61,608 61,888

Dongsishitiao (DSST) 28,337 29,699

Dongzhimen (DZM) 68,115 76,757

Fuchengmen (FCM) 40,719 41,256

Fuxingmen (FXM) 25,738 25,965

Guomao (GM) 37,648 42,513

Haidianhuangzhuang (HDHZ) 16,530 24,348

Huixingxijienankou (HXXJNK) 15,917 15,931

Jishuitan (JST) 42,887 41,686

Jianguomen (JGM) 19,172 20,078

Jiaomenxi (JMX) 15,654 15,373

Sanyuanqiao (SYQ) 21,642 26,334

Wukesong (WKS) 22,802 19,444

Songjiazhuang (SJZ) 36,461 34,595

Xierqi (XEQ) 47,962 47,204

Xizhimen (XZM) 61,966 59,315

Xuanwumen (XWM) 22,397 21,278

the alighting passenger demands as shown in Fig. 2(a). Unlike the WKS station, the DZM station connects an important long distance bus hub where passengers are transferred to long-distance coaches to other cities in China. This trend became obvi ous during traditional Chinese holidays, such as Spring Festival and Mid-Autumn Festival. On September 29, 2012 (the day before the Mid-Autumn Festival), a vast number of passengers returned to their hometowns for reunions and were trans ferred at the DZM station for other surface transportation modes. Compared with other normal Saturdays, the return flow caused a sudden rise in alighting passenger demands for the entire day. Fig. 2(b) reveals that passenger demands increased by more than 40%. Different from the previous two scenarios (i.e., special events held adjacent to the WKS station and DZM station), a subway service disruption caused by a train signal failure occurred at approximately 7:00 PM on August 23, 2012 at the HDHZ station. The HDHZ station is an interchange station connecting lines 4 and 10 and is located in the mixed land use area surrounded by universities, academic institutes, residence communities, and shopping malls. The disruption stranded a large number of passengers and forced these passengers to change their routes by using buses and taxis, leading to a sudden rise of egressing passenger flows. Given that no prior warning information was released for the emergency evac uation, this can be considered an unplanned event. The events that happened at the WKS and DZM stations were known in advance. Thus, the sudden increase of passenger flows was expected to some extent. However, detecting the exact time when the irregular passenger flows arise and pinpointing the possible stations that generate the most incoming passengers for the target stations are difficult. Compared with planned events, predicting an unplanned surge of passenger flow because of special events is more challenging and useful because these events do not receive sufficient treatment or attention before they actually occur. Moreover, no external information, such as event location, time, and type, is available as prior knowledge to estimate the potential impact.

To forecast the non-regular passenger demands on these three stations, the boarding passenger flows from the remaining 18 stations are considered as possible contributors to the sudden passenger flow increase and incorporated into the predic tion model as inputs. The previous time series of alighting passenger flows at the same stations also serve as inputs. The rationale of choosing 18 stations is that the incoming passenger flow at each station ranks in top 18 among 321 subway sta tions, but these stations contribute approximately 27% of total boarding demands on a daily basis. Incorporating additional stations will significantly increase the complexity of the proposed algorithm. Therefore, a tradeoff between accuracy and effi ciency is made. We aim to determine the dynamics of passenger flows between the remaining stations and the target sta tions and pinpoint the stations that are most vulnerable for special attention because these stations contribute more passenger flows to the WKS, DZM, and HDHZ stations.

3. Methodology

3.1. The NARX model

To forecast the passenger flow of a target subway station, a multi-input and single-output (MISO) nonlinear system iden tification and parametric modeling problem is considered. A general form of a discrete-time Non-linear AutoRegressive Mov ing Average with eXogenous inputs (NARMAX) model is widely applied to construct the MISO system, which can be described by the following nonlinear recursive difference formula (Leontaritis and Billings, 1985; Wei et al., 2004a):

310 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328800 Alighting

Volume

Jul.21,2012 Jul.14,2012 Jul.28,2012 Aug.4,2012 Aug.11,2012 Aug.18,2012 Aug.25,2012 Sep.1,2012

700

600

500

400

300

200

100

00:00 6:00 12:00 18:00 00:00 0 Time

(a)

3000 Alighting

Volume

Sep.29,2012 Aug.11,2012 Aug.18,2012 Aug.25,2012 Sep.1,2012 Sep.8,2012 Sep.15,2012 Sep.22,2012

2500

2000

1500

1000

500

00:00 6:00 12:00 18:00 00:0 0 Time

(b)

2500 Alighting

Volume

Aug.23,2012 Jul.12,2012 Jul.19,2012 Jul.26,2012 Aug.2,2012 Aug.9,2012 Aug.16,2012 Aug.30,2012

2000

1500

1000

500

0

00:00 6:00 12:00 18:00 00:00

Time

(c)

Fig. 2. Alighting passenger demand fluctuation at WKS, DZM, and HDHZ subway stations: (a) a concert was held at 7:30 PM on July 21, 2012 (Planned event); (b) passengers returned home before the Mid-Autumn Festival on September 29, 2012 (Planned event); (c) subway service disruption happened on August 23, 2012 (Unplanned event)

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 311

yðtÞ ¼ fðyðt   1Þ; ... ; yðt   nyÞ; uiðt   1Þ; ... ; uiðt   nuÞ;eðt   1Þ; ... ;eðt   neÞÞ þ eðtÞ ð1Þ

where y(t) refers to the alighting passenger flow at the target station at time step t, uiðtÞ refers to the boarding passenger flows at station i at time step t, eðtÞ represents the model error at time step t, and f indicates an unknown linear or nonlinear mapping function and usually links the system output to the system inputs between the available inputs and outputs. ny, nu, and ne indicate the maximum time lags in the previous outputs, inputs, and the model error, respectively. The unit time lag in this study is 15 min with the same sampling rate of subway passenger flows. The comprehensive description of the NAR MAX methods of dynamical systems is derived from Wei and Billings (Wei et al., 2004a).

For many practical problems, the nonlinear function fð Þ in Equation (1) is generally unknown, and the most commonly employed approach to approximate the unknown function fð Þ is to adopt a polynomial NARMAX model. To simplify the problem, the NARX model, a subset of the NARMAX model without considering the moving average noise terms in Eq. (1), is applied in our cases. The NARX model can be described in Eq. (2):

yðtÞ ¼ fðyðt   1Þ; ... ; yðt   nyÞ; uiðt   1Þ; ... ; uiðt   nuÞÞ þ eðtÞ ð2Þ

Define q ¼ ny þ nu and xðtÞ¼½x1ðtÞ; x2ðtÞ; ... ; xqðtÞ  with

xkðtÞ ¼ yðt   kÞ 1 6 k 6 ny

ð3Þ

uðt   ðk   nyÞÞ ny þ 1 6 k 6 ny þ nu

Thus, the NARX model can be defined in a linear parameter form as follows:

yðtÞ ¼ ^fðxðtÞÞ þ eðtÞ ¼ XM m¼1

hm/mðxðtÞÞ þ eðtÞ ¼ uT ðtÞh þ eðtÞ ð4Þ

where M represents the total number of regressor terms, hm represent the model coefficients, and /mðxðtÞÞ ðm ¼ 1; 2; ... ; MÞ are the model regressors. uðtÞ¼½/1ðxðtÞ; ... ;/MðxðtÞÞÞ T and h indicate the regressor matrix and the parameter matrix respectively.

In this study, we will adopt a novel MSRBF network model with Gaussian kernels to approximate nonlinear function ^f. The related content will be provided in the next section.

3.2. MSRBF network

Generally, the MSRBF network includes both small and large scales in the network in a hierarchical multiscale manner. Thus, it can accommodate both the local and global properties of the basis functions. In the multiscale modeling scheme, multiple kernel widths or different scale parameters are assigned to each basis function. A block diagram of an MSRBF net work is shown in Fig. 3. How to construct the network structure and determine the centers and scales in the multiscale mod eling framework are discussed in the following section.

3.2.1. Constructing the MSRBF network structure

The MSRBF network of a nonlinear dynamical system is represented as

yðtÞ ¼ Xq

k¼1

where hðlinearÞ

k xkðtÞ þXI

hðlinearÞ

i¼0

XJ j¼0

XNc

hðRBFÞ

i;j;m ui;j;mðxðtÞ; cm; sði;jÞ

m Þ ð5Þ

m¼1

k and hðRBFÞ

i;j;m are the unknown parameters to be estimated, and ui;j;mðxðtÞ; cm; sði;jÞ

m Þ is the mth Gaussian basis func

tion that is defined by

2 "

m Þ ¼ exp  Xny

nXyþnu

235 ð6Þ

ui;j;mðxðtÞ; cm; sði;jÞ

k¼1

xkðtÞ cm;k sðiÞ

y;m

k¼nyþ1

xkðtÞ cm;k sðjÞu;m

where xðtÞ¼½x1ðtÞ; ... ; xqðtÞ  given in (3) is the network input vector, cmðtÞ¼½cm;1; ... ; cm;q  is the center vector of the mth basis function, and sði;jÞ

m is the scale of the mth basis function in the network. sði;jÞ

m is represented as

sði;jÞ

zfflfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflfflffl{

zfflfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflfflffl{

m ¼ sðiÞ

y;m; ... ; sðiÞy;m

1:ny ; sðjÞu;m; ... ; sðjÞu;m

1:nu

ð7Þ

Note that Nc is the number of centers or basis functions in the MSRBF network, and (I + 1) and (J + 1) denote the number of scales for the output and input variables in the mth center respectively. Therefore, for a single-input and single-output sys tem, a total of M ¼ ðI þ 1ÞðJ þ 1ÞNc basis functions are included in the network. Particularly, for a MISO system, the number of basis functions for the input variables is ðJ þ 1Þn, where n is the number of input variables. Thus, the network contains a total of M ¼ ðI þ 1ÞðJ þ 1ÞnNc basis functions.

312 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

Input layer

Hidden layer 1 c 2 c q c

1,1 s 1,2 s 1 1,i s 2,2 s 2,1 s 2 2,i s q,1 s q,2 s , q qi s

Output layer

-

Fig. 3. Schematic of the MSRBF network.

3.2.2. Determination of the center

All measured observations can be regarded as candidate kernel centers cm when the length of the measured dataset is not very long. In comparison, if a long dataset is considered and all the observations are still regarded as candidate kernel cen ters, then a great number of model terms or regressors will be included in the initial MSRBF network. Thus, the training of the network will be time-consuming. Recently, to resolve this problem, a sample of a learning algorithm based on the orthogonal least squares method was proposed to choose optimal centers one by one until an adequate network is constructed and each selected center maximizes the increment to the energy of the desired output (Chen et al., 1991). This method provided a simple but efficient way for choosing centers and avoiding numerical ill-conditioning. Shi et al. (2005) adopted sensitivity analysis to construct RBF networks, where the number of hidden neurons and the centers in the network were determined by the minimal number of hidden neurons with the maximal sensitivity and the maximization of the output’s sensitivity to the training data respectively.

Generally, the well-known k-means clustering algorithm, which assumed that each data vector belongs exclusively to a single cluster, was applied to select the centers in the network (Moody and Darken, 1989). By contrast, in practical applica tion, data vectors commonly belong to more than one cluster and are associated with each element as a set of membership level (Li et al., 2016a). Thus, the fast fuzzy c-means (FFCM) clustering algorithm was used to significantly reduce the number of candidate centers of the basis function in the network (Biniaz and Abbasi, 2013). The FFCM algorithm, coupled with the partition index criterion proposed by Bensaid et al. (1996a), is briefly introduced below.

In the FFCM clustering algorithm, the objective function with respect to fuzzy membership lm;j and cluster centers cm is defined by

Jc ¼ XNk m¼1

XN

lcm;jkxj   cmk2 ð8Þ j¼1

where xjðj ¼ 1; 2; ... ;NÞ is the jth input vector, N is the length of the input vector, Nk is the number of clusters, c > 1 is the fuzziness index, and k k is the Euclidean norm.

The procedure of updating centers in the FFCM algorithm is briefly described as follows:

Step1: For q-dimensional input vector, rearrange lmj to Nk   N dimension matrix.

Step2: Let new fuzzy membership be l mj and label matrix L ¼ fL1; L2; ... ; LNk g, where Lk is the label vector of the kth clus ter in the current iteration.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 313

Step3: Denote all data points corresponding to Lk label matrix be Ik.

Step4: Define Ik ¼ Ik1; Ik2; ... ; Ikcnkfor kth cluster, where cnk is the number of data points in the kth cluster. Step5: Update the center of the kth cluster using the following formula:

Pcnk

i¼1Iki

c k ¼

cnkð9Þ

Similar to the k-means clustering algorithm, the number of centers should also be given first in the FFCM algorithm. Some criteria, including partition coefficient, classification entropy, and partition index, can be applied to choose the optimal clus ters. The partition coefficient and classification entropy are not directly connected to a geometrical property (Xie and Beni, 1991). Hence, the partition index criterion, which is a sum of the individual cluster validity measures normalized through division by the fuzzy cardinality of each cluster, is adopted to determine the optimal clusters. The optimal value of Nk is the value that minimizes the partition index (SC) as follows (Bensaid et al., 1996b):

PN

SCðNkÞ ¼ XNk m¼1

j¼1lcm;jkxj   cmk2

k¼1kcm   ckk2 ð10Þ -mPNk

where -m is the fuzzy cardinality of cluster m defined as -m ¼ PNj¼1lm;j.

The above FFCM clustering algorithm with partition index can be applied to select the number of centers in the MSRBF network. For a given training dataset of length N, let Nc ¼ arg minNk fSCðNkÞg. Thus, the Nc candidate centers (commonly Nc   N) will be at least included in the MSRBF network, which can be determined by a fast fuzzy c-mean clustering algo rithm. Generally, the conventional k-means clustering algorithm can only obtain a locally optimal solution, which mainly depends on the initial locations in the cluster centers. To overcome this problem, some improved clustering algorithms including the FFCM is employed in this study (Biniaz and Abbasi, 2013).

3.2.3. Determination of the scales

For the given Nc pairs of input–output measurements fuðtÞ; yðtÞgNt¼1, let ru and ry be the standard derivation of the input vector fuðtÞgNt¼1 and output vector fyðtÞgNt¼1 respectively. The scale vector in (7) can be defined by (Billings et al., 2007) sðiÞ

y;m ¼ ba iry; i ¼ 0; 1; ... ; I ð11Þ

sðjÞu;m ¼ ba jru; j ¼ 0; 1; ... ; J ð12Þ

where m ¼ 1; 2; ... ;Nc and a > 1 are constants. A substantial number of experiments have shown that a good selection for the constants a and b is to assign a ¼ 2 and 1 6 b 6 3 (Wei et al., 2004a).

Let }3 ¼ fui;j;mð ; dði;jÞ

m ; cmÞg with i ¼ 0; ... ; I; j ¼ 0; ... ; J; m ¼ 1; ... ;Nc . The triple-indexed set }3 is indicated as the dic tionary associated with the new MSRBF networks. To simplify the descriptions, redefine the elements of }3 so that the triple index ði; j; mÞ can be represented by a single index m ¼ 1; 2; ... ; M; where M is the number of basis functions included in the network, to form a single-index dictionary }1 ¼ /mð Þ : /m 2 }3 f ; m ¼ 1; 2; ... ; Mg.In this study, two types of dictionaries }1 and }3 will not be distinguished, and both dictionaries will be used to indicate a uniform symbol }. Thus, the MSRBF network in (5) can be represented by

^fðxðtÞÞ ¼ XM m¼1

hm/mðxðtÞÞ ð13Þ

The derivation discussed in this section can easily be described as a multiple-input and single-output case, which will be discussed in Section 4.

3.3. Model structure selection and the MPOLS algorithm

In the nonlinear system identification and modeling procedure, model structure selection or model subset detection is usually a key step (Li et al., 2016c). It commonly consists of selecting first the significant model regressor terms from a redundant candidate model term, and then setting the significant regressor terms to construct a parsimonious final model. As shown in the later examples, numerous candidate model terms or basis functions may be involved in the MSRBF network (13) when the parameters I, J, and Nc are large or the number of input variable n is large. However, many of these candidate model terms may be redundant, and only a subset of these model terms is significantly included in the model (Li et al., 2016b, 2012). In particular, these redundant candidate model terms may lead to a large number of parameters for estimation in the model. Consequently, the redundant model may become oversensitive to the training dataset and may exhibit poor generalization properties (Wei et al., 2004b). Therefore, detecting which candidate terms should be contained in the MSRBF model is crucial. In the present study, an efficient MPOLS algorithm (Billings and Wei, 2005), which is a simple and relatively

314 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

computationally efficient regressor selection algorithm, is employed to solve the model structure detection problem for the MSRBF network models. The detailed description of the MPOLS algorithm can be found in Appendix A. Once the significant model terms are chosen, we can easily verify that the relationship between the selected original basis a1; a2; ... ; an and the associated orthogonal bases w1; w2; ... ; wn are represented as

An ¼ WnUn ð14Þ

where An ¼ ½a1; ... ; an , Wn is an N   n orthogonal matrix with columns w1; w2 ... ; wn, and Un is an n   n unit upper trian gular matrix with entries that are calculated from the orthogonalization procedure. The unknown parameter vector, which is denoted by hn ¼ ½h1; ... ; hn T for the model from the original bases, can then be obtained from the triangular equation Unhn ¼ gn with gn ¼ ½g1; g2; ... ; gn T and gk ¼ ðyTwkÞ=ðwTkwkÞ.

3.4. Multi-step-ahead forecasting

Multi-step ahead forecasting is a challenging task in time-series prediction because of the increasing number of uncer tainties from various sources, such as the lack of information and accumulated errors (Sorjamaa et al., 2007). The common goal of multi-step ahead time-series forecasting task is to predict the values of yðt þ sÞ with s P 1 using a historical time series fyðtÞgNt¼1 included N observed sequences. To achieve this goal, a model or a predictor is commonly constructed from the available data. To obtain the multi-step-ahead predictors of nonlinear time series, the iterative approach is adopted (Wei and Billings, 2006). Considering the s-step-ahead forecasting problem, we first forecast the first step by adopting the model. The forecasted value is applied as a part of the input variables for the next step prediction. Thus, the entire horizon can be repeatedly predicted step by step.

The task for s-step-ahead forecasts is constructing a model that can predict the value of yðt þ sÞ by adopting a set of selected variables fyðtÞ; yðt   1Þ; ... ; yðt   d þ 1Þg. Let the trained one-step-ahead model be fð1Þ ¼ f yð ðtÞ; yðt   1Þ; ..., yðt   d þ 1Þ; thus, the forecasts are defined by (Ben Taieb et al., 2012)

(

yðt þ sÞ ¼ fðsÞðy^ðt þ 1Þ; ... ; y^ðt þ s   1Þ;yðN   d þ sÞ; ... ; yðtÞÞ; s 2 f2; ... ; dg

ð15Þ

fðsÞðy^ðt þ s   dÞ; ... ; y^ðt þ s   1ÞÞ; s 2 fd þ 1; ... ; sg

where fðsÞ with s P 1 are nonlinear functions, and d is the model order or maximum lag. In practical applications, the non linear function fðsÞ is generally unknown and complex. Therefore, a class of flexible models, which exhibit excellent approx imation capabilities and can represent a broad class of highly complex systems, is required to achieve multi-step-ahead predictions. The model that employs MSRBF as the basis function to approximate the s-step predictor fðsÞ will be investigated in this study as a new approach to obtain accurate s-step predictions.

4. Experimental result

In the experiments, three scenarios with extreme high passenger flows from three subway stations in a Beijing subway network are employed to illustrate the applicability and effectiveness of the newly proposed MSRBF network for nonlinear system modeling compared with that of state-of-the-art prediction algorithms, including SVM, boosted regression tree (BRT), and single-scale RBF (SSRBF) networks. The mean absolute percentage error (MAPE), variance of absolute percentage error (VAPE), and root mean square error (RMSE) will be used to evaluate the performance of the associated models. The prediction performance is evaluated by adopting the defined MAPE, VAPE, and RMSE indices (Balestrassi et al., 2009; Boto-Giralda et al., 2010; Sun et al., 2015)

MAPE ¼ 1NXN i¼1

jyi   y^ij

y    100% ð16Þ

VAPE ¼ 1NXN i¼1

r

ðei    eÞ2   100% ð17Þ

ffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffiffi

RMSE ¼

1

N

XN

i¼1ðyi   y^iÞ2

ð18Þ

where N is the number of observational samples, y ¼ fy1; y2; ... ; yNg, y^ ¼ fy^1; y^2; ... ; y^Ng, and y  ¼ 1NPNi¼1yi represent the observations of the system output, the model predictions, and the average of the observations, respectively. ei ¼ jðyi   y^iÞj=yi and  e ¼ 1NPNi¼1ei indicate the absolute error and the mean of absolute error respectively.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 315

4.1. Example 1: WKS subway station

In this subsection, WKS station, which is the target station for prediction, was considered a structure-unknown (black box) dynamical system. The objective was to construct a mathematical model that can be applied to forecast the alighting passenger flow of WKS station on July 21, 2012. The detailed data description is presented in Section 2. The subway system was considered an 18-input and single-output system, whereas the alighting passenger flow of WKS station (circled in red) was the system output. The boarding passenger flows of 18 transfer stations (circled in other colors) in Fig. 1 were the system inputs. The smart card data were recorded from 4:45 am to 23:15 pm with a sampling rate of 15 min. Thus, 76 data points were obtained on a daily basis, resulting in a total of 15 day observations and 15   76 ¼ 1140 data points, as indicated in Fig. 4. The first 1064 data points were used for model identification, and the remaining 76 data points were employed for model prediction. The objective was to conduct iterative multistep predictions using the proposed MSRBF modeling approach.

For the convenience of description, let yW ðtÞ represent the alighting passenger flows of WKS target station, and u1ðtÞ; ..., u18ðtÞ indicate the boarding passenger flows from 18 original transfer stations in Beijing subway network, respectively. The next step is to determine the maximum time lags of inputs (i.e., ny and nu) shown in Equation (2). The time series of alighting passengers at WKS station was considered, and yW ðt   1Þ was selected as the primary contributor to the current alighting passenger flow, indicating ny ¼ 1. The selection of nu is based on the shortest commuting times from the 18 transfer stations to the targeted WKS station. The passengers are assumed to choose these routes with the least number of transfers, and the commuting time between any two stations will be estimated by counting both in-train travel time and transfer time, as presented in Fig. 5. For example, transferring from FXM station to WKS station requires 30 min, which is equivalent to two time lags considering the sample rate of 15 min. That is to say, the boarding passenger flows at FXM station 30 min ago are unlikely to contribute to the alighting passenger flows at WKS station, and should not be included in the prediction model. However, the passengers boarding from the FXM station may take longer than 30 min to arrive at the WKS station because each passenger’s actual travel time between the two stations may exceed the minimum commuting time (i.e., 30 min). To incorporate the late arrivals, a buffer of 30 min is added to the shortest commuting time, which is equivalent to two time lags. Therefore, the maximum time lag of boarding passenger flows from FXM station can be computed as the shortest com muting time (30 min) plus the buffer time (30 min) divided by the sampling rate (15 min), thus yielding four in this scenario. This result implies the boarding passenger flows at time steps t   2;t   3;t   4 are incorporated in the model, but the pas senger flow at time step t   1 cannot influence the alighting passenger flow at WKS station because passengers still transfer from FXM station to WKS station. Based on a similar logic, the maximum time lags for the remaining 17 stations can be cal culated. The results demonstrate that each station will contribute their boarding passenger flows with three time lags to the alighting passenger flow at WKS station. Therefore, a total of 55 candidate variables were chosen. The variables contain the alighting passenger flow at WKS station at time step t   1 and the boarding passenger flows at the remaining 18 stations, including three time lags according to the example calculation process. The variables are considered potential contributors for constructing the MSRBF network by using the MPOLS algorithm.

The contribution of the 55 candidate variables will unlikely be equal in the identified model. Some of the variables may be insignificant, and the associated model terms will be automatically removed from the model. The ERR criterion is a very use ful index to indicate the significance of model terms defined in Appendix A. The model eventually includes only several sig nificant terms, where most of the terms slightly contribute to the system output and can thus be disregarded. The GCV criteria, which is an effective model validation criteria for selecting the number of model terms, together with ERR criteria,

Volume

1200

1000

800

600

400

200

7/7 7/14 7/210

Date

Fig. 4. Original transfer passenger flow time series at WKS station from July 7 to July 21, 2012. (Blue line is training data, Red line is predicted data). (For interpretation of the references to colour in this figure legend, the reader is referred to the web version of this article.)

316 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

Time (min)

Target Station : DZM

60

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Consuming Time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS | CYM | JGM | DSST | FCM | DWL | GM | JST H | XXJNK | XZM | XWM | FXM | SYQ | SJZ | WKS | HDHZ | JMX | XEQ |  |  |

50

40

30

20

10

0

Transfer Station

Fig. 5. Consumption time from the 18 transfer stations to the WKS target station.

are used to select the significant terms in the initial MSRBF network. Therefore, 11 significant variables were chosen from a total of 55 candidate variables (Wei et al., 2004b). The input vector for the MSRBF network model was chosen to be xðtÞ¼½x1ðtÞ; x2ðtÞ; ... ; x11ðtÞ  ¼ ½ ð yW ðt   1Þ; u1 t   4Þ; u4ðt   2Þ; u6ðt   2Þ; u6ðt   4Þ; u8ðt   2Þ; u8ðt   4Þ, u11ðt   5Þ; u14ðt   3Þ; u16ðt   4Þ; u17ðt   4Þ . The selected model variables with estimated parameters and ERR values are listed in Table 2.

The fast fuzzy c-means clustering algorithm was applied in the training dataset, and the SC index given in Eq. (10) sug gests that the optimal number of clusters Nc for this dataset was 15. The 1064 observational data samples were partitioned into 15 groups, and the centers of the 15 groups were chosen as the candidate centers for constructing the MSRBF network. The basis functions in the MSRBF network model (6) were given by

!2 2

4  X11

m Þ ¼ exp   x1ðtÞ   cm;1

!23 xkðtÞ   cm;k

ui;j;mðxðtÞ; cm; sði;jÞ

sðiÞyw;m

k¼2

sðjÞu;m

5 ð19Þ

where m ¼ 1; 2; ... ; 15 and the mth scale vector sði;jÞ

m is represented by

sði;jÞ

zfflfflfflfflfflffl}|fflfflfflfflfflffl{

zfflfflfflfflfflffl}|fflfflfflfflfflffl{

i

m ¼ sðiÞ

yw;m; sðjÞu1;m; sðjÞu4;m; sðjÞu6;m; sðjÞu6;m

1:2; sðjÞu8;m; sðjÞu8;m

1:2

sðjÞu11;m; sðjÞu14;m; sðjÞu16;m; sðjÞu17;m

ð20Þ

According to (11) and (12), parameters a and b; I; J in the scale sði;jÞ

m from the MSRBF network were chosen as 2, 2, 1, and 1,

respectively.

Table 2

Selected variables, estimated parameters, and the corresponding ERR values in

Example 1.

Search steps Terms Estimates of parameters ERRs ( 100%)

1 yw(t   1) 0.5196 9.631E+001

2 u16 (t   4) 0.0878 3.885E 001

3 u4 (t   2) 0.1631 6.870E 001

4 u8 (t   2) 0.1500 8.450E 002

5 u1 (t   4)  0.1037 2.605E 001

6 u17 (t   4) 1.0811 9.199E 002

7 u6 (t   2) 0.4681 1.213E 001

8 u6 (t   4)  0.5132 2.365E 001

9 u14 (t   3) 0.1073 5.580E 002

10 u8 (t   4)  0.1346 7.849E 002

11 u11 (t   5) 0.0537 2.356E 002

Note: yw, u4, u16, u14, u6, u12, u7, and u18 indicate the passenger flow of the WKS

target station, and JST, HXXJNK, DZM, JMX and DWL, XZM, and XEQ are the

respective transfer stations.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 317

Thus, the initial network model involves a total of 11 þ 211   15 ¼ 30731 candidate model regressor terms. The MPOLS algorithm was adopted to the 30,731 candidate regressor terms over the 1064 training data points. According to the value of GCV, 67 significant regressor terms were selected and adopted to build the final MSRBF network for forecasting the time series of passenger flow.

The validation dataset was applied to test the performance of the identified model, which consists of 76 data points of alighting passenger flow at WKS station on July 21, 2012. When a concert was held at 19:00 near the WKS target station, the passenger flow suddenly increased. Multi-step ahead prediction was discussed in Section 3. The one-step ahead predic tion is equivalent to 15-min-ahead prediction because the interval time is 15 min. To allow a more proactive action to mit igate the adverse effect of a sudden increase of passenger flow, two-step ahead prediction is conducted. This process will notify both transit operators and travelers 30 min ahead of an extreme event occurrence in a timely fashion. Both one step ahead and two-step ahead predictions are calculated from the identified 67-term MSRBF model and other conventional methods, such as SVM, BRT, and SSRBF network. All the comparative methods adopted the same 55 candidate variables as input matrix. The results of SVM were acquired by using the LIBSVM package, where the optimal parameters were obtained by particle swarm optimization algorithm. The BRT algorithm was implemented by following the work of (Hara and Chellappa, 2013) with the optimal number of trees. The scale of SSRBF was also optimally determined. The performance comparisons of both one-step-ahead and two-step-ahead (TSA) prediction results with different models are presented in Figs. 6 and 7.

The results show that the proposed MSRBF network exhibited a superior performance to predict non-passenger flow. A demand peak was observed at around 18:30 PM in WKS station because of the concert. The sudden increase of passenger flow could be successfully identified by the proposed algorithm. Fig. 6 indicates that the MSRBF network outperforms other methods with a closer estimate of passenger flows with special events. Even for a more rigorous TSA prediction test, the pro posed MSRBF network still exhibits the best prediction performance compared with other algorithms, as indicated in Fig. 7.

Fig. 8 presents the histograms of the alighting passenger flow error measures obtained by the proposed MSRBF network and conventional methods for one-step-ahead and TSA predictions. The proposed MSRBF network generates the lowest fore casting errors (MAPE, VAPE, and RMSE) compared with the other approaches, thus demonstrating the effectiveness of the proposed method.

The statistical results in Table 3 indicate that the proposed MSRBF network consistently outperforms other approaches with the lowest MAPE, VAPE, and RMSE values for both one-step-ahead and TSA predictions. For example, MAPE, VAPE, and RMSE values from the proposed MSRBF network are 11.8038, 1.5910, and 62.7863 for one-step-ahead predictions, and 15.1206, 3.2279, and 85.3464 for TSA predictions, respectively. These metrics are extremely lower than those of other methods, such as SVM, BRT, and SSRBF. Therefore, these results quantitatively indicate that the proposed MSRBF network effectively predicts a sudden increase in subway passenger flow.

One advantage of the proposed MSRBF network is that the structure of the MSRBF network involves both linear and non linear parts, which can effectively describe the stochastics and dynamic relationship for nonlinear systems. Although the nonlinear part may increase computational complexity, the MPOLS algorithm is applied to eliminate most redundant terms for less computational burdens. SVM also performs well in predicting small-scale datasets. However, the proposed MSRBF network functions well for both small-scale and large-scale datasets. Unlike the traditional SSRBF network, the MSRBF network accommodates both local and global properties of the basis functions to include both small and large scales in

Volume

700 600 500 400

Real Value SVM BRT SSRBF MSRBF

300 200 100 0

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:45 Time

Fig. 6. One-step-ahead prediction results of alighting passenger flow at WKS station on July 21, 2012.

318 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

700

600

Real Value SVM BRT SSRBF MSRBF

Volume

500 400 300 200 100 0

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:45 Time

Fig. 7. Two-step-ahead prediction results of alighting passenger flow at WKS station on July 21, 2012.

MAPE

16 14 12 10 8

6

4

2

VAPE

4

3.5 3

2.5 2

1.5 1

0.5

(a)

RMSE

100 80

60

40

20

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

MAPE

25 20 15 10 5

VAPE

(b)

7

6

5

4

3

2

1

RMSE

140 120 100 80

60

40

20

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

Fig. 8. Three-measure values from four forecasting methods at WKS target station: (a) one-step-ahead predictions, (b) two-step-ahead predictions.

the network, and thus yield better prediction performance. The proposed MSRBF network generates the optimal forecasting performance, particularly on predicting unusual passenger flow, which likely implies the occurrence of a special event, such as a concert or sport game.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 319

Table 3

Predictions of passenger flow at WKS target station on July 21, 2012.

Methods One-step-ahead Two-step-ahead

MAPE VAPE RMSE MAPE VAPE RMSE

SVM 15.7608 3.8876 91.7416 22.0388 6.4300 122.1048 BRT 14.5227 1.6327 70.1059 17.3865 3.2573 89.3661 SSRBF 13.5816 2.4339 75.1777 18.0762 5.7126 108.9132 MSRBF 11.8038 1.5910 62.7863 15.1206 3.2279 85.3464

Note: The bold values indicate the minimum error rate from Example 1.

4.2. Example 2: DZM subway station

Unlike the WKS station, the alighting passenger volume at DZM station on September 29, 2012 significantly increased for the entire day compared with the usual weekends, as indicated in Fig. 2(b). This result occurred because most passengers returned home before the Mid-autumn festival via the long-distance bus hub adjacent to the station. Similar to Example 1, the subway system was also considered an 18-input and single-output system. Let system output variable yDðtÞ be the alighting passenger flow at DZM station, and the remaining 18 main transfer stations u01ðtÞ; ... ; u018ðtÞ are the boarding pas senger flows as system input variables. Therefore, the 1140 data points from September 15 to September 29 can be visual ized in Fig. 9.

The shortest commuting time between DZM subway station and other 18 transfer stations is calculated in Fig. 10. Similar to example 1, 55 candidate variables, yDðt   jDÞ with jD ¼ 1, u0kðt   j1Þ with k ¼ 1; 2; 3 and j1 ¼ 1; 2; 3, u0kðt   j2Þ with k ¼ 4; ... ; 12 and j2 ¼ 2; 3; 4, u0kðt   j3Þ with k ¼ 13; ... ; 17 and j3 ¼ 3; 4; 5, u0kðt   j4Þ with k ¼ 18 and j4 ¼ 4; 5; 6, were selected. Both the GCV and ERR criteria were applied to tune up the 12 most significant terms among the initial 55 variables into the prediction model. Therefore, the input vector for the MSRBF network model can be represented as xðtÞ¼½x1ðtÞ; x2ðtÞ; ... ; x12ðtÞ  ¼ yD ½ ðt   1Þ;u01ðt   1Þ; u02ðt   3Þ; u03ðt   1Þ; u08ðt   2Þ; u08ðt   4Þ; u09ðt   4Þ; u013ðt   3Þ; u013ðt   4Þ; u015ðt   3Þ; u017ðt   3Þ; u017ðt   5Þ . The selected model variables with the estimated parameters and ERR values are shown in Table 4.

Applying the fast fuzzy c-means clustering algorithm to cluster the centers leads to the optimal number of clusters Nc , which is five. The training dataset is partitioned into five groups, and the basis functions in the MSRBF network model (6)

are given by

" !2

m Þ ¼ exp   x1ðtÞ   cm;1

X12

!23 xkðtÞ   cm;k

ui;j;mðxðtÞ; cm; sði;jÞ

sðiÞyD;m

k¼2

sðjÞu;m

5 ð21Þ

where m ¼ 1; 2; ... ; 5, and the mth scale vector sði;jÞ

"

sði;jÞ

zfflfflfflfflfflffl}|fflfflfflfflfflffl{

m is defined by zfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflffl{

zfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflffl{

#

m ¼ sðiÞ

yD;m; sðjÞu01;m; sðjÞu02;m; sðjÞu03;msðjÞu08;m; sðjÞu08;m

1:2;

sðjÞu09;m; sðjÞu013;m; sðjÞu013;m

1:2; sðjÞu015;m; sðjÞu017;m; sðjÞu017;m

1:2

ð22Þ

Volume

3500

3000

2500

2000

1500

1000

500

9/15 9/22 9/290

Date

Fig. 9. Original transfer passenger flow time series at DZM station from September 15–29, 2012. (Blue line is training data, and red line is predicted data). (For interpretation of the references to colour in this figure legend, the reader is referred to the web version of this article.)

320 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

Time (min)

Target Station : DZM

60

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Consuming Time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS | CYM | JGM | DSST | FCM | DW | LGM | JST H | XXJN | KXZ | M XW | MFX | MSYQ | SJZ | WK | S HDH | Z JM | X XE | Q |  |

50

40

30

20

10

0

Transfer Station

Fig. 10. Real consumption time between DZM subway station and other 18 subway stations.

Table 4

Selected model terms, estimated parameters, and ERR values in Example 2.

Search steps Term Estimates of parameters ERRs ( 100%)

1 yDðt   1Þ 0.7541 9.754e+001

2 u08ðt   4Þ  0.1193 7.735e 001

3 u08ðt   2Þ 0.3999 5.166e 001

4 u013ðt   3Þ 2.6696 9.999e 002

5 u015ðt   3Þ 0.1299 1.172e 001

6 u09ðt   4Þ  0.3400 4.845e 002

7 u017ðt   3Þ 0.7057 4.825e 002

8 u03ðt   1Þ 0.3398 4.023e 002

9 u02ðt   3Þ  0.1945 2.793e 002

10 u017ðt   5Þ  0.5443 3.405e 002

11 u01ðt   1Þ 0.0811 2.859e 002

12 u013ðt   4Þ  1.2956 1.346e 002

Note: yDðtÞ; u08ðtÞ; u013ðtÞ; u015ðtÞ; u09ðtÞ; u017ðtÞ; u03ðtÞ; u02ðtÞ; u01ðtÞ represent the pas

senger flows of DZM, JST, SYQ, WKS, HXXJNK, JMX, JGM, CYM, and BS stations,

respectively.

According to (11) and (12), the scales of the MSRBF network sði;jÞ

a; b; I; J are also selected to be 2, 1, 1, and 1.

m can be calculated, which is similar to example 1, where

The initial network model involves 12 linear terms and 5   212 ¼ 20ð;Þ480 nonlinear terms in total. The MPOLS algorithm is applied to select the most important terms over 20,492 terms. A total of 36 significant terms are selected according to the value of the GCV criteria. The 36 terms are adopted to form the final MSRBF network to forecast the passenger flow series. A total of 76 samples on September 29, 2012 are used for the one-step ahead and two-step ahead predictions. The comparison results from four methods are shown in Figs. 11 and 12.

Fig. 11 shows that the MSRBF network yields more accurate results than the SSRBF network and the BRT model and sig nificantly outperforms SVM. The trend of abnormal subway passenger flows can be accurately captured by the proposed MSRBF network. To further demonstrate the effectiveness of the proposed algorithm, multi-step-ahead prediction is under taken to prevent the model from achieving good performance for one-step-ahead prediction, although the model is inferior. The comparison results from different forecasting methods are shown in Fig. 12. Similar to the outcome for one-step-ahead prediction, the proposed MSRBF network still exhibited the best performance among all the tested methods, whereas the prediction accuracy the SSRBF and BRT decreased. These results are further confirmed by the quantitative measures in Fig. 13 and Table 5, which indicate that the MAPE, VAPE, and RMSE values from the proposed MSRBF network are signifi cantly lower than those of the other conventional prediction methods. The proposed algorithm can accurately detect the occurrence of a special event at least 30 min in advance, thus saving a considerable amount of time for information dissem ination and crowd control.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 321

Volume

2500 2000 1500 1000 500

Real Value SVM BRT SSRBF MSRBF

0

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:45 Time

Fig. 11. One-step-ahead prediction results of alighting passenger flow at DZM station on Sept. 29, 2012.

Volume

2500 2000 1500 1000 500

0

Real Value SVM BRT SSRBF MSRBF

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:45

Time

Fig. 12. Two-step-ahead prediction results of alighting passenger flow at DZM station on Sept. 29, 2012.

4.3. Example 3: HDHZ subway station

Concerts events or returning home before the festival are categorized as planned events. Example 3 is set as an unplanned event to demonstrate the superiority of the proposed method further. The alighting passenger volume at HDHZ station on August 23, 2012 increased in the afternoon unlike that on usual weekdays, as presented in Fig. 2(c). This event occurred because the train signal failed for a while and the passengers remained at the HDHZ station. Similar to the previous exam ples, the HDHZ subway system was an 18-input and single-output system. Let yHðtÞ be the alighting passenger flow at HDHZ station and u 1ðtÞ; ... ; u 18ðtÞ be the boarding passenger flows from the remaining 18 transfer stations. The 1140 data points from August 9–23 are presented in Fig. 14.

Fig. 15 shows the shortest commuting time between HDHZ subway station and other 18 transfer stations. The 55 candidate variables of the system, yHðt   pHÞ with pH ¼ 1, u kðt   p1Þ with k ¼ 1; 2; ... ; 10 and p1 ¼ 2; 3; 4, u kðt   p2Þ with k ¼ 11; ... ; 16 and p2 ¼ 3; 4; 5, u kðt   p3Þ with k ¼ 17; 18. The initial 10 significant terms were determined by the GCV and ERR criteria, and the input vector for the MSRBF network model can be represented as

xðtÞ¼½x1ðtÞ; x2ðtÞ; ... ; x10ðtÞ  ¼ yHðt   1Þ; u 1ðt   2Þ; u 1ðt   4Þ; u 8ðt   3Þ; u 9ðt   3Þ; u 9ðt   4Þ; u 16ðt   3Þ; u 16ðt   4Þ; u 18ðt   5Þ;  u 18ðt   6Þ . The selected model variables with estimated parameters and ERR values are presented in Table 6.

322 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

MAPE MAPE

30 25 20 15 10 5

0

40 30 20 10 0

SVM BRT SSRBF MSRBF

SVM BRT SSRBF MSRBF

VAPE VAPE

6

5

4

3

2

1

0

10 8

6

4

2

0

(a)

SVM BRT SSRBF MSRBF  (b)

SVM BRT SSRBF MSRBF

RMSE RMSE

600 500 400 300 200 100 0

700 600 500 400 300 200 100 0

SVM BRT SSRBF MSRBF

SVM BRT SSRBF MSRBF

Fig. 13. Three measures from four methods at DZM target station: (a) one-step-ahead predictions, and (b) two-step-ahead predictions.

Table 5

Predictions of passenger flow at DZM station on Sept. 29, 2012.

Methods One-step-ahead Two-step-ahead

MAPE VAPE RMSE MAPE VAPE RMSE

SVM 29.6094 5.5889 514.0743 39.9336 9.1452 679.6365 BRT 11.2654 0.8311 196.5157 13.7130 1.3375 243.2507 SSRBF 9.5299 0.5431 162.9400 35.0508 6.9167 594.5454 MSRBF 8.2634 0.4411 143.8376 13.3725 1.2359 235.9457

Note: The bold values indicate the minimum error rate from Example 2.

The optimal number of clusters Nc is 29, which was obtained by the fast fuzzy c-means clustering algorithm. Therefore, the training data set is partitioned into 29 groups. The basis functions in the MSRBF network model (6) were given by

" !2

m Þ ¼ exp   x1ðtÞ   cm;1

X10

!23 xkðtÞ   cm;k

ui;j;mðxðtÞ; cm; sði;jÞ

sðiÞ

yD;m

k¼2

sðjÞu;m

5 ð23Þ

where m ¼ 1; 2; ... ; 29 and the mth scale vector sði;jÞ m

"

sði;jÞ

zfflfflfflfflfflffl}|fflfflfflfflfflffl{

zfflfflfflfflfflffl}|fflfflfflfflfflffl{

zfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflffl{

zfflfflfflfflfflfflfflffl}|fflfflfflfflfflfflfflffl{

#

m ¼ sðiÞ

yH;m; sðjÞu 1;m; sðjÞu 1;m

1:2;

sðjÞu 8;m; sðjÞu 9;m; sðjÞu 9;m

1:2; sðjÞu 16;m; sðjÞu 16;m

1:2; sðjÞu 18;m; sðjÞu 18;m

1:2

ð24Þ

The scales of the MSRBF network sði;jÞ

m can be calculated by Eqs. (11) and (12), and parameters a; b; I; J are selected to be 2,

2, 1, and 1.

The initial network model of HDHZ subway station involves a total of 10 linear terms and 29   210 ¼ 29; 696 nonlinear terms. Similar to the previous examples, the MPOLS algorithm is utilized to eliminate the redundant terms of over 29,706

Volum

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 323

1800

1600

1400

1200

1000

800

600

400

200

0

8/9 8/15 8/23

Date

Fig. 14. Original transfer passenger flow time series at HDHZ station from August 9–23, 2012. (Blue line is training data, and red line is predicted data). (For interpretation of the references to colour in this figure legend, the reader is referred to the web version of this article.)

Target Station : HDHZ

Time (min)

60

|  |  |  |  |  | Consuming Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FCM HX |  | XJN | KJST | XZM | DZM | FXM | SYQ | WK |  | S XE | Q XWM | BS | CYM | DSST | GM | JGM | JMX | DW | LSJZ |  |

50

40

30

20

10

0

Transfer Station

Fig. 15. Real consumption time between HDHZ subway station and other 18 subway stations. Table 6

Selected model terms, estimated parameters, and ERR values in Example 3.

Search steps Term Estimates of parameters ERRs( 100%) 1 u 16ðt   4Þ 0.3434 9.572e+001 2 u 8ðt   3Þ 0.0307 1.9115e+000 3 u 18ðt   6Þ  0.2457 2.789e 001 4 yHðt   1Þ 0.3580 4.896e 001 5 u 16ðt   3Þ 0.6018 1.588e 001 6 u 9ðt   4Þ  0.0216 5.048e 002 7 u 1ðt   4Þ 0.1019 9.345e 002 8 u 18ðt   5Þ 0.1628 2.280e 002 9 u 9ðt   3Þ  0.0571 2.012e 002 10 u 1ðt   2Þ 0.0410 1.690e 002 Note: u 16ðtÞ; u 8ðtÞ; u 18ðtÞ; yHðtÞ; u 9ðtÞ; u 1ðtÞ represent the passenger flows of the JMX, WKS, SJZ, HDHZ, XEQ, and FCM stations, respectively.

324 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

Volume

1800 1600 1400 1200 1000

800 600

Real Value SVM BRT SSRBF MSRBF

400 200 0

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:45 Time

Fig. 16. One-step-ahead prediction results of alighting passenger flow at HDHZ station on Aug. 23, 2012.

Volume

2000 1800 1600 1400 1200 1000 800

600

Real Value SVM BRT SSRBF MSRBF

400 200

4:45 6:45 8:45 10:45 12:45 14:45 16:45 18:45 20:45 22:450

Time

Fig. 17. Two-step-ahead prediction results of alighting passenger flow at HDHZ station on Aug. 23, 2012.

terms, and 19 significant terms are selected according to the value of the GCV criteria. The 19 terms are adopted to form the final MSRBF network to forecast the passenger flow series on August 23, 2012. The 76 samples on that day are used for the one-step ahead and two-step ahead predictions. The comparison results from four methods are shown in Figs. 16 and 17 respectively.

The one-step-ahead prediction and TSA prediction are presented in Figs. 16 and 17 respectively. The figures show that the MSRBF network yields the result of unplanned event more exactly than the tested methods BRT, SVM, and SSRBF network in

Table 7

Predictions of passenger flow at HDHZ station on Aug. 23, 2012.

Methods One-step-ahead Two-step-ahead

MAPE VAPE RMSE MAPE VAPE RMSE

SVM 16.8591 3.3995 97.5344 19.7495 3.5522 106.5762 BRT 11.2488 1.7529 67.8242 12.1775 1.7550 70.2490 SSRBF 10.6231 0.8299 54.6337 25.6854 5.0596 133.2901 MSRBF 9.0696 0.8280 50.1554 11.0555 1.5470 64.9659

Note: The bold values indicate the minimum error rate from Example 3.

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 325

MAPE

18 16 14 12 10 8

6

4

2

VAPE

3.5 3

2.5 2

1.5 1

0.5

(a)

RMSE

100 80

60

40

20

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

MAPE

30 25 20 15 10 5

VAPE

(b)

6

5

4

3

2

1

RMSE

140 120 100 80

60

40

20

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

SVM BRT SSRBF MSRBF 0

Fig. 18. Three measures from four methods at HDHZ target station: (a) one-step-ahead predictions, and (b) two-step-ahead predictions.

the one-step-ahead prediction and TSA prediction. This result demonstrates that the proposed MSRBF network could deter mine and reflect the complex relationship between the subway stations and exhibit superiority in forecasting multi-step ahead prediction.

To illustrate the superiority of the proposed method, the results are further confirmed by the quantitative measures in Table 7 and Fig. 18. The MAPE, VAPE, and RMSE values from the proposed MSRBF network are lower than those of the other conventional prediction methods, which are significantly lower than those of SVM and SSRBF. Compared to the BRT method, which both shows a stable performance on one-step-ahead prediction and TSA prediction, the results of proposed MSRBF method are more accurate.

The proposed MSRBF network exhibited the best forecasting performance among the four methods, particularly in pre dicting the unusual passenger flow. The occurrence of planned events, such as concert or sport game, and the unplanned events, such as train signal failure, could both be forecasted 30 min earlier.

5. Conclusions

Predicting subway disruptions caused by special events is a well-known challenge in transportation planning and oper ation, because non-regular passenger demands are very difficult to determine using traditional statistical methods or machine learning approaches. A timely and reliable detection method of sudden demand change is crucial for transit oper ators to disseminate evaluation information to the crowd and allocate resources (e.g., feeder and shuttle buses) to transport heavy passenger flows in subway systems. In addition, understanding the relationship between incoming passenger flows at the network level and the demand surge at a particular station can assist governmental agencies in policy making. For exam ple, subway operators may strategically close certain subway stations to limit passenger crowding before special events, and the government can adopt a more flexible subway fare to reduce passenger demands.

Most existing literature focuses on regular passenger demand forecasting in short- or long-prediction horizons with very limited studies on subway passenger flow prediction under special events. In this paper, a novel MSRBF network approach is proposed for forecasting the short-term passenger flow (30 min ahead). The results obtained with the proposed methodol ogy using multi different scales are compared with conventional techniques, such as BRT, SVM, and SMRBF. The proposed approach for predicting short-term passenger flow outperforms these techniques, particularly when an emergent event

326 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

occurs. The proposed model involves two parts. The linear submodel can be used to track the linear relationship between the output and the inputs, and the MSRBF submodel can be applied to capture the effect of the nonlinear dynamics, which may not be completely explained by the linear submodel. This measure effectively enhances the capability of predicting short term passenger flow.

The main disadvantage of the proposed model approach is that a large number of candidate terms may be involved in the initial model when the number of system input vectors is large. To address this problem, an effective MPOLS algorithm is employed to select the most significant candidate terms to construct a parsimonious linear parameters model. Thus, the problem is resolved successfully. The results of this study provide a new way to predict the short-term passenger flow, which is based on the parameter model. The proposed MSRBF network modeling framework, together with the MPOLS algorithm, can be improved further to achieve better prediction effect.

The methodology proposed in this study was developed for multi-step ahead forecasting results obtained only from the passenger flow of a Beijing subway network. The degree of generality of the algorithm cannot be assessed at present. Future studies can investigate the performance of the proposed method on different benchmark datasets and some practical appli cations on real time-series forecasts using the promising MSRBF network approach. Another interesting future direction is to incorporate the transfer passenger demands from adjacent bus stops. This treatment is expected to enhance prediction accu racy from the perspective of multimodal transportation systems.

Acknowledgement

This work was supported in part by the National Natural Science Foundation of China [61671042, 61403016, 51408019, U1564212, 71402011], the Science and Technology Research Foundation for Transportation [2015318221020], the Special ized Research Fund for the Doctoral Program of Higher Education [20131102120008], the Beihang University Innovation and Practice Fund [YCSJ-02-2015-12], and the Beijing Nova Program [z151100000315048].

Appendix A

The MPOLS algorithm

Let y ¼ ½yð1Þ; ... ; yðNÞ T be a vector of observational outputs at N time instants and um ¼ ½/mð1Þ; ... ; /mðNÞ T be a vector formed by the mth candidate model term with m ¼ 1; 2; ... ; M. Meanwhile, let } ¼ fu1; ... ;uMg be a dictionary with the M candidate bases. A large number of modeling experiences and practical modeling identifications showed that finite dimen sional set } is usually redundant. The model term selection problem is equivalent to finding, from M candidate model terms, a full dimensional subset }n ¼ fa1; ... ; ang¼fui1; ... ;uin g of n bases where n 6 M. Thus, y can be approximated by using a linear parameter combination of a1; a2 ... ; an as below

y ¼ h1a1 þ ... þ hnan þ e ð25Þ or in a compact matrix expression

y ¼ Ah þ e ð26Þ

where the matrix A ¼ ½a1; ... ; an  is a full column rank, h ¼ ½h1; ... ; hn  is a coefficient vector, and e is the approximation error. The procedure of model structure selection starts in (25) to find vector u‘1 from the candidate regressor family fu1; ... ;uMg. Thus, u‘1 is the ‘‘best” matching regressor to y, namely, u‘1 with the following linear regression: yðtÞ ¼ amumðtÞ þ nmðtÞ ð27Þ The minimum of the mean squared error can be achieved as presented in (28).

XN

n2‘1ðtÞ ¼ 1NXN

( )

1

N

t¼1

t¼1

ðyðtÞ   a‘1u‘1ðtÞÞ2 ¼ minm1NXN t¼1

½yðtÞ   amumðtÞ 2

ð28Þ

The ‘‘best” matching regressor term u‘1 can be found by using an orthogonal projection method that is defined by cos v ¼ yT

yTy pum ffiffiffiffiffiffiffiffiffiffiffiffiffiffi

ffiffiffiffiffiffiffiffi

p ð29Þ uTmum

ku1mk¼kyk cos v ¼ yTum ffiffiffiffiffiffiffiffiffiffiffiffiffiffi

p ð30Þ uTmum

such that

n2mðtÞ¼knmk2 ¼ kyk2   ku1mk2 ¼ yTy   ðyTumÞ2

XN

uTmumð31Þ

t¼1

Thus,

( )

‘1 ¼ arg max 16m6MðyTumÞ2

uTmum

Y. Li et al. / Transportation Research Part C 77 (2017) 306–328 327ð32Þ

The first significant basis can be selected as a1 ¼ u‘1, and the first associated orthogonal basis can be selected as w1ðtÞ ¼ u‘1ðtÞ. Set g1 ¼ ðyTw1Þ=ðwT1w1Þ, ERR1 ¼ g21ðwT1w1Þ

ðyT yÞ , and e1ðtÞ ¼ yðtÞ   g1w1ðtÞ.Similarly, in the second step, find a vector

u‘2 from the candidate regressor term family fum : 1 6 m 6 M; m–‘1g. Thus, u‘2 is the ‘‘best” matching regressor term to e1. According to (27) and (28), ‘2 can be obtained as follows:

( )

‘2 ¼ arg max 16m6M;m–‘1

ðeT1umÞ2 uTmum

ð33Þ

Thus, the second significant basis can be chosen as a2ðtÞ ¼ u‘2ðtÞ, and the second associated orthogonal basis can be selected by

w2 ¼ a2   wT1a2

wT1w1w1 ð34Þ

and set g2 ¼ ðyTw2Þ=ðwT2w2Þ, ERR2 ¼ g22ðwT2w2Þ=ðyTyÞ, e2ðtÞ ¼ e1ðtÞ   g2w2ðtÞ.

The kth significant model term can be selected as

( )

‘k ¼ arg max 16m6M;

m–‘1;

ðeTk 1ukÞ2 uTkuk

ð35Þ

Set akðtÞ ¼ u‘kðtÞ, and orthogonalize ak with w1; w2 ... ; wk 1 as below

wk ¼ ak   wT1ak

wT2w2w2       wTk 1ak

wT1w1w1   wT2ak

ERRk ¼ g2kðwTkwkÞ

wTk 1wk 1wk 1 ð36Þ

ðyTyÞ ð37Þ

and calculate gk ¼ ðyTwkÞ=ðwTkwkÞ, and ekðtÞ ¼ ek 1ðtÞ   gkwkðtÞ. Note that the ek can be used to form a criteria for model selection, and the ERRk is just equal to the error reduction ratio, which provides a simple but effective means for obtaining a subset of significant regressors. According to the value of ERRk, the significant terms can be selected in a forward-regression manner. The detailed procedure was discussed by Billings and Wei (2005).

The procedure of model term selection can be terminated when some desired termination conditions are met. In this study, the generalized cross-validation (GCV) criterion (Billings et al., 2007) is adopted to determine the model size    2

GCVðnÞ ¼ N N   kn

MSEðnÞ ð38Þ

where k ¼ maxf1; qNg and 0 6 q 6 0:01. The mean-square-error (MSE) in (38) is represented by MSE ¼ PNt¼1½yðtÞ   y^ðtÞ 2=N, where y^ðtÞ is the one-step ahead prediction produced from the associated n term model. The model selection procedure will be terminated at the step when the index function GCV(n) is minimized.

References

Balestrassi, P.P., Popova, E., Paiva, A.P., Lima, J.W.M., 2009. Design of experiments on neural network’s training for nonlinear time series forecasting. Neurocomputing 72, 1160–1178.

Ben Taieb, S., Bontempi, G., Atiya, A.F., Sorjamaa, A., 2012. A review and comparison of strategies for multi-step ahead time series forecasting based on the NN5 forecasting competition. Expert Syst. Appl. 39, 7067–7083.

Bensaid, A., Hall, L., Bezdek, J., 1996a. Validity guided reclustering with applications to image segmentation. IEEE Trans. Fuzzy Syst. 4, 112–122. Bensaid, A.M., Hall, L.O., Bezdek, J.C., Clarke, L.P., Silbiger, M.L., Arrington, J.A., Murtagh, R.F., 1996b. Validity-guided (re)clustering with applications to image segmentation. IEEE Trans. Fuzzy Syst. 4.

Billings, S.A., Wei, H.L., 2005. The wavelet-NARMAX representation: a hybrid model structure combining polynomial models with multiresolution wavelet decompositions. Int. J. Syst. Sci. 36, 137–152.

Billings, S.A., Wei, H.L., Balikhin, M.A., 2007. Generalized multiscale radial basis function networks. Neural Networks 20, 1081–1094. Biniaz, A., Abbasi, A., 2013. Fast FCM with spatial neighborhood information for brain Mr image segmentation. J. Artif. Intell. Soft Comput. Res. 3, 15–26. Boto-Giralda, D., Diaz-Pernas, F.J., Gonzalez-Ortega, D., Diez-Higuera, J.F., Anton-Rodriguez, M., Martinez-Zarzuela, M., Torre-Diez, I., 2010. Wavelet-based

denoising for traffic volume time series forecasting with self-organizing neural networks. Comput Aided Civ. Infrastruct. Eng. 25, 530–545. Chan, S., Miranda-Moreno, L., 2013. A station-level ridership model for the metro network in Montreal, Quebec. Can. J. Civ. Eng. 40, 254–262. Chen, S., Cowan, C.N., Grant, P.M., 1991. Orthogonal least squares learning algorithm for radial basis function networks. IEEE Trans. Nerual Networks 2. 302- 209.

de Gooijer, J.G., Hyndman, R.J., 2006. 25 years of time series forecasting. Int. J. Forecast. 22, 443–473.

328 Y. Li et al. / Transportation Research Part C 77 (2017) 306–328

Hara, K., Chellappa, R., 2013. Computationally efficient regression on a dependency graph for human pose estimation. Comput. Vis. Pattern Recog. 9, 3390– 3397.

Horowitz, A.J., 1984. Simplifications for single-route transit-ridership forecasting models. Transportation 12, 261–275. Jiang, X., Zhang, L., Chen, X.M., 2014. Short-term forecasting of high-speed rail demand: a hybrid approach combining ensemble empirical mode decomposition and gray support vector machine with real-world applications in China. Transport. Res. Part C Emerg. Technol. 44, 110–127. Karlaftis, M.G., Vlahogianni, E.I., 2011. Statistical methods versus neural networks in transportation research: differences, similarities and some insights. Transport. Res. Part C-Emerg. Technol. 19, 387–399.

Khashei, M., Bijari, M., Hejazi, S.R., 2012. Combining seasonal ARIMA models with computational intelligence techniques for time series forecasting. Soft. Comput. 16, 1091–1105.

Kuppam, A., Copperman, R., Lemp, J., Rossi, T., Livshits, V., Vallabhaneni, L., Jeon, K., Brown, E., 2013. Special events travel surveys and model development. Transport. Lett. 5, 67–82.

Leontaritis, I.J., Billings, S.A., 1985. Input-output parametric model for non-linear systems. Part I: Deterministic non-linear systems. Int. J. Control 41, 303– 328.

Li, Y., Liu, Q., Tan, S.R., Chan, R.H.M., 2016a. High-resolution time-frequency analysis of EEG signals using multiscale radial basis functions. Neurocomputing 195, 96–103.

Li, Y., Luo, M.L., Li, K., 2016b. A multiwavelet-based time-varying model identification approach for time-frequency analysis of EEG signals. Neurocomputing 193, 106–114.

Li, Y., Wei, H.L., Billings, S.A., 2011a. Identification of time-varying systems using multi-wavelet basis functions. IEEE Trans. Control Syst. Technol. 19, 656– 663.

Li, Y., Wei, H.L., Billings, S.A., Liao, X.F., 2012. Time-varying linear and nonlinear parametric model for Granger causality analysis. Phys. Rev. E 85. Li, Y., Wei, H.L., Billings, S.A., Sarrigiannis, P.G., 2011b. Time-varying model identification for time-frequency feature extraction from EEG data. J. Neurosci. Methods 196, 151–158.

Li, Y., Wei, H.L., Billings, S.A., Sarrigiannis, P.G., 2016c. Identification of nonlinear time-varying systems using an online sliding-window and common model structure selection (CMSS) approach with applications to EEG. Int. J. Syst. Sci. 47, 2671–2681.

Long, Y., Thill, J., 2015. Combining smart card data and household travel survey to analyze jobs–housing relationships in Beijing. Comput. Environ. Urban Syst. 53, 19–35.

Ma, X.L., Tao, Z.M., Wang, Y.H., Yu, H.Y., Wang, Y.P., 2015. Long short-term memory neural network for traffic speed prediction using remote microwave sensor data. Transport. Res. Part C-Emerg. Technol. 54, 187–197.

Ma, X.L., Wang, Y.H., Chen, F., Liu, J.F., 2012. Transit smart card data mining for passenger origin information extraction. J. Zhejiang Univ. Sci. C-Comput. Electr. 13, 750–760.

Ma, X.L., Wu, Y.J., Wang, Y.H., Chen, F., Liu, J.F., 2013. Mining smart card data for transit riders’ travel patterns. Transport. Res. Part C Emerg. Technol. 36, 1– 12.

Moody, J., Darken, C.J., 1989. Fast learning in networks of locally-tunned processing units. Neural Comput. 1, 281–294. Ni, M., He, Q., Gao, J., 2017. Forecasting the subway passenger flow under event occurrences with social media. IEEE Trans. Intell. Transport. Syst. (in press). Pereira, F.C., Rodrigues, F., Ben-Akiva, M., 2015. Using data from the web to predict public transport arrivals under special events scenarios. J. Intell. Transport. Syst. 19, 273–288.

Rojas, I., Valenzuela, O., Rojas, F., Guillen, A., Herrera, L.J., Pomares, H., Marquez, L., Pasadas, M., 2008. Soft-computing techniques and ARMA model for time series prediction. Neurocomputing 71, 519–537.

Shi, D., Yeung, D.S., Gao, J., 2005. Sensitivity analysis applied to the construction of radial basis function networks. Neural Networks 18, 951–957. Si, B., Fu, L., Liu, J., Shiravi, S., Gao, Z., 2015. A multi-class transit assignment model for estimating transit passenger flows—a case study of Beijing subway network. J. Adv. Transport.

Sorjamaa, A., Hao, J., Reyhani, N., Ji, Y.N., Lendasse, A., 2007. Methodology for long-term prediction of time series. Neurocomputing 70, 2861–2869. Sun, Y.X., Leng, B., Guan, W., 2015. A novel wavelet-SVM short-time passenger flow prediction in Beijing subway system. Neurocomputing 166, 109–121. Taylor, B.D., Miller, D., Iseki, H., Fink, C., 2009. Nature and/or nurture? Analyzing the determinants of transit ridership across US urbanized areas. Transport. Res. Part A Policy Pract. 43, 60–77.

Tsai, T.H., Lee, C.K., Wei, C.H., 2009. Neural network based temporal feature models for short-term railway passenger demand forecasting. Expert Syst. Appl. 36, 3728–3736.

Vlahogianni, E.I., Karlaftis, M.G., Golias, J.C., 2014. Short-term traffic forecasting: where we are and where we’re going. Transport. Res. Part C Emerg. Technol. 43, 3–19.

Wang, X., An, K., Tang, L., Chen, X., 2015. Short term prediction of freeway exiting volume based on SVM and KNN. Int. J. Transport. Sci. Technol. 4, 337–352. Wei, H.L., Billings, S.A., 2006. An efficient nonlinear cardinal B-spline model for high tide forecasts at the Venice Lagoon. Nonlinear Proc. Geophys. 13, 577– 584.

Wei, H.L., Billings, S.A., Balikhin, M., 2004a. Prediction of the DST index using multiresolution wavelet models. J. Geophys. Res. Space Phys. 109. Wei, H.L., Billings, S.A., Liu, J., 2004b. Term and variable selection for non-linear system identification. Int. J. Control 77, 86–110. Xie, X.L., Beni, G., 1991. A validity measure for fuzzy clustering. IEEE Trans. Pattern Anal. Mach. Intell. 13, 841–847. Xue, R., Sun, D., Chen, S.K., 2015. Short-term bus passenger demand prediction based on time series model and interactive multiple model approach. Discr. Dynam. Nat. Soc.

Zhang, D.P., Wang, X.K., 2014. Transit ridership estimation with network Kriging: a case study of Second Avenue Subway, NYC. J. Transp. Geogr. 41, 107–115. Zhang, G.Y., Wu, Y.G., Liu, Y.Q., 2014. An advanced wind speed multi-step ahead forecasting approach with characteristic component analysis. J. Renew. Sustain. Energy 6.

Zhong, C., Batty, M., Manley, E., Wang, J., Wang, Z., Chen, F., Schmitt, G., 2016. Variability in regularity: mining temporal mobility patterns in London, Singapore and Beijing using smart-card data. PLoS One 11, e0149222.

Zhong, C., Manley, E., Arisona, S.M., Batty, M., Schmitt, G., 2015. Measuring variability of mobility patterns from multiday smart-card data. J. Comput. Sci. 9, 125–130.

Zhou, J., Murphy, E., Long, Y., 2014. Commuting efficiency in the Beijing metropolitan area: an exploration combining smartcard and travel survey data. J. Transp. Geogr. 41, 175–183.